Purposes:

- Establishing a new benchmark in large-sample hydrological modeling using the VIC model.

- Evaluating calibration performance using additional observational resources for streamflow prediction and facilitating knowledge transfer from gauged to ungauged basins.

Scenario 1 : calibration based on streamflow show the performance of other hydrologic variables (SM, water storage, evapotranspiration, or etc) and then calibration based other hydrologic variables in ungauged basins and evaluate streamflow , transfer knowledge of routing parameters through Regionalization

Scenario 2: Scenario 2, transfer routing parameters through Random forest

Scenario 3: Transfer all the parameters to the ungauged basins through ML models and physical and geographical features

- Assimilating USGS streamflow observations into the VIC model across 517 basins to generate an hourly hydrological dataset encompassing all key hydrological variables.

KGE median : 0.7232821620032044

- Greater value of binf yields lower infiltration and more runoff (Qd)

- Amount of infiltration capacity relative to the saturated gridcell area

Baseflow

- Linear at low soil moisture content

– Reduces responsiveness of baseflow during dry conditions

- Non-linear at high soil moisture content

– Rapid baseflow response during wet conditions

B= Ds. DsmaxWs Wnc Wn Linear base flow

Ds - [>0 to 1] This is the fraction of Dsmax where non-linear (rapidly increasing) baseflow begins. With a higher value of Ds, the baseflow will be higher at lower water content in lowest soil layer.

Dsmax - [>0 to ~30, depends on hydraulic conductivity] This is the maximum baseflow that can occur from the lowest soil layer (in mm/day).

Ws - [>0 to 1] This is the fraction of the maximum soil moisture (of the lowest soil layer) where non-linear baseflow occurs. This is analogous to Ds. A higher value of Ws will raise the water content required for rapidly increasing, non-linear baseflow, which will tend to delay runoff peaks.

binf - [>0 to ~0.4] This parameter defines the shape of the Variable Infiltration Capacity curve. It describes the amount of available infiltration capacity as a function of relative saturated gridcell area. A higher value of binf gives lower infiltration and yields higher surface runoff.

Variable infiltration curve parameter (binfilt)

Fraction of Dsmax where non-linear baseflow begins

Maximum velocity of baseflow (maximum baseflow)

Fraction of maximum soil moisture where non-linear baseflow occurs

Exponent n in Campbell's eqn for hydraulic conductivity

Soil Depth